52000*48PECIFICATIONS FOR MODEL GX-2009 PERSONAL MONITOR

- Instrument must simultaneously detect up to 4 gases

- Instrument must display all four gas sensor readings simultaneously on LCD display

- Instrument must detect Combustible gases from 0-100% LEL

- Instrument must detect Oxygen from 0-40%

- Instrument must detect Hydrogen Sulfide 0-100ppm

- Instrument must detect Carbon Monoxide 0-500 ppm

- Instrument must have at least 3 alarms on each channel. Alarm 1, Alarm 2, and Over range Alarm

- Audible alarm must have3 at least two output ports to prevent muffling of alarm

- Instrument must also have TWA and STEL alarms on H2S and CO

- Instrument must have 3 alarm warnings, Audible, Visual and Vibrating

- Instrument must not weigh more than 5 Ounces

- Instrument must include protective rubber overmolding

- Instrument must be rated and certified as IP-67 or better

- All Sensors must weigh less than 0.3 ounces combined

- CO sensor must have replaceable interference filter

- Sensors must not use pin/socket connection for electrical interface

- Instrument must not be larger than 2.7" H x 3.0" W x 1" D

- Instrument backlight must automatically come on with alarm

- Instrument must datalog up to 300 hours of readings.

- Datalog down load must be via IRDA port

- Instrument must have peak hold method to review peaks on the instrument

- Instrument must operate at least 20 hours on a charge

- User button must glow in the dark

- Standard instrument must sample by diffusion, but have optional automatic and manual sample pump adapters

- Instrument must also be capable of showing current time

- Instrument must use integral Ni-MH rechargeable batteries able to be recharged in 3 hours or less

- Battery charger cradle must be capable of accepting 100-240VAC or 12VDC vehicle adapters

- Optional expandable Gang Charger must be available

- Instrument must come standard with an alligator clip capable of rotating 360 deg., and have optional belt clip

- Instrument must have a 2 year warranty on sensors and electronics.

- Instrument must have autocalibration feature, with option for individual sensor calibration

- Instrument must be compatible with optional docking station for calibration, bump testing, charging, and datalog download

- Unit must be approved as intrinsically safe for use in Class I Group A, B, C, and D hazardous locations to both US and Canadian standards, and also be performance tested and certified to CSA standard C22.2 No. 152

- Instrument must be approved as intrinsically safe for use in Zone 0 Exia IIC hazardous locations and carry ATEX/IECEX/CE certification